UC Test Case Dataset Summary

Seven Network Unit Commitment Instances for Algorithm Scaling Studies

Topology: IEEE benchmarks  |  UC parameters: Kazarlis (1996), RTS-GMLC (2019), UCjl (2020)

# 1.  Dataset Overview

Seven network UC instances are provided, covering a 50× range in system size (6 to 300 buses) and a 80× range in peak demand (283 MW to 23,525 MW). The first three cases (6-bus, IEEE 14, IEEE 39) form the primary experimental set. The remaining four (IEEE 30, 57, 118, 300) form the scaling study set.

| Test case | Buses | Lines | Gens | RES | Peak demand | Installed cap. | RES cap. | Topology source | UC params source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6-bus (Wood & Wollenberg) | 6 | 9 | 3 | 2 | 286 MW | 430 MW | 280 MW | Wood & Wollenberg (1996) | Kazarlis (1996) |
| IEEE 14-bus | 14 | 20 | 6 | 2 | 648 MW | 902 MW | 570 MW | Christie (1993) | Kazarlis (1996) + RTS-GMLC |
| IEEE 39-bus (New England) | 39 | 46 | 10 | 4 | 2,656 MW | 7,010 MW | 1,900 MW | Athay (1979), Pai (1989) | Kazarlis (1996) + RTS-GMLC |
| IEEE 30-bus | 30 | 41 | 6 | 2 | 283 MW | 435 MW | 74 MW | Christie (1993) | UCjl / Xavier (2020) |
| IEEE 57-bus | 57 | 76 | 7 | 3 | 1,251 MW | 1,180 MW | 276 MW | Christie (1993) | UCjl / Xavier (2020) |
| IEEE 118-bus | 118 | 120 | 54 | 4 | 4,242 MW | 12,885 MW | 986 MW | Christie (1993) | UCjl / Xavier (2020) |
| IEEE 300-bus | 300 | 309 | 69 | 5 | 23,525 MW | 16,195 MW | 2,908 MW | IEEE Task Force (1993) | UCjl / Xavier (2020) |

RES = Renewable Energy Sources (wind + solar PV). Installed cap. = sum of all generator Pmax. All cases use T=24 h horizon, carbon price = 50 $/tCO₂, VoLL = 20,000 $/MWh.

# 2.  Parameter Methodology

## 2.1  Network topology and generator capacities

The network topology (buses, lines, susceptances, thermal limits) and generator capacity limits (Pmin, Pmax) are taken verbatim from the standard IEEE benchmark sources listed in Section 5. No modifications are made to these elements.

## 2.2  UC parameters — primary cases (6-bus, IEEE 14, IEEE 39)

Thermal generator UC parameters are drawn from Kazarlis et al. (1996) [1], the canonical 10-unit UC benchmark, matched by technology type and capacity class. Costs are scaled linearly by Pmax / Pmax_ref. Nuclear unit parameters follow RTS-GMLC / Barrows et al. (2019) [2]: UT = DT = 168 h, startup cost = 500 $/MW × Pmax. Hydro parameters also follow RTS-GMLC [2]: UT = DT = 1 h, zero startup/shutdown/no-load costs.

## 2.3  UC parameters — scaling cases (IEEE 30, 57, 118, 300)

UC parameters for the scaling cases follow the methodology of UnitCommitment.jl (Xavier et al., 2020) [3], which is published under a modified BSD license with a citable Zenodo DOI. The specific rules applied are:

- Ramp-up, ramp-down, startup rate, and shutdown rate: set to 30% of Pmax per hour (UCjl fixed proportion rule).

- Minimum up/down times (UT/DT): assigned by technology class using Kazarlis type mapping (coal large: 8/8 h, coal mid: 6/6 h, CCGT: 3/3 h, OCGT/diesel: 1/1 h).

- Fuel costs: assigned per technology type consistent with Kazarlis cost ordering.

- No-load, startup, and shutdown costs: scaled from Kazarlis reference values by Pmax ratio.

## 2.4  Demand profiles

All demand profiles are synthetic 24-hour normalized load curves following a standard diurnal shape, scaled to the system peak of each benchmark. Bus-level demand is distributed proportionally to the base-case MW loads published in MATPOWER.

## 2.5  Renewable units

Renewable units (wind and solar PV) are added to each case at selected buses. Wind availability follows a diurnal profile consistent with NREL ATB archetypes [4]. Solar PV follows a standard bell-curve irradiance profile. Curtailment cost is set to 5 $/MWh for all units.

# 3.  New Scaling Case-by-Case Summaries

## 3.1  IEEE 30-bus (scaling)

Entry point for the scaling study. Six generators of mixed type on a 41-line network. System peak is comparable to the 6-bus case but with five times the network complexity. Two renewable units (wind + solar, 74 MW combined peak) provide moderate renewable penetration.

| Parameter | Value |
| --- | --- |
| Buses / Lines | 30 / 41 |
| Thermal generators | 6 (2 Coal-mid, 2 CCGT, 2 OCGT) |
| Renewable units | 2 (Wind 60 MW, Solar 40 MW) |
| Peak system demand | 283.4 MW |
| Total installed capacity | 435 MW |
| Renewable penetration (peak) | 36.1% |
| UC param source | UCjl methodology — Xavier et al. (2020) [3] |
| Topology source | Christie (1993), PSTCA [6] |

## 3.5  IEEE 57-bus (scaling)

Mid-size scaling case. Seven generators on a 76-line network with a 1,251 MW peak demand. Three renewable units (two wind, one solar, 276 MW combined) provide roughly 22% renewable penetration at peak. The MATPOWER case57 fmax values are used from the pglib-opf corrected version.

| Parameter | Value |
| --- | --- |
| Buses / Lines | 57 / 76 |
| Thermal generators | 7 (3 Coal, 2 CCGT, 2 OCGT) |
| Renewable units | 3 (2 Wind, 1 Solar — total 276 MW peak) |
| Peak system demand | 1,251.0 MW |
| Total installed capacity | 1,180 MW |
| Renewable penetration (peak) | 22.1% |
| UC param source | UCjl methodology — Xavier et al. (2020) [3] |
| Topology source | Christie (1993), PSTCA [6] |

## 3.6  IEEE 118-bus (scaling)

Large-scale case with 54 generators and 120 lines. The 54-generator fleet creates a rich UC problem. Four renewable units (two wind, two solar, 986 MW combined) provide approximately 23% penetration. This case is the standard in recent scalability papers for network UC.

| Parameter | Value |
| --- | --- |
| Buses / Lines | 118 / 120 |
| Thermal generators | 54 (classified by Pmax into CCGT/OCGT/Diesel) |
| Renewable units | 4 (2 Wind, 2 Solar — total 986 MW peak) |
| Peak system demand | 4,242.0 MW |
| Total installed capacity | 12,885 MW |
| Renewable penetration (peak) | 23.2% |
| UC param source | UCjl methodology — Xavier et al. (2020) [3] |
| Topology source | Christie (1993), PSTCA [6] |

## 3.7  IEEE 300-bus (scaling)

The largest case in the dataset. 69 generators on a 309-line network with a 23,525 MW system peak — approximately 90× larger than the 6-bus case. Five renewable units (three wind, two solar, 2,908 MW combined) provide 12% renewable penetration. This case tests algorithm scalability at transmission-system scale.

| Parameter | Value |
| --- | --- |
| Buses / Lines | 300 / 309 |
| Thermal generators | 69 (mix of coal-large, CCGT, OCGT) |
| Renewable units | 5 (3 Wind, 2 Solar — total 2,908 MW peak) |
| Peak system demand | 23,525.0 MW |
| Total installed capacity | 16,195 MW |
| Renewable penetration (peak) | 12.4% |
| UC param source | UCjl methodology — Xavier et al. (2020) [3] |
| Topology source | IEEE Task Force (1993) [9] |

# 4.  Ready-to-Paste Paper Section

We evaluate the proposed method on seven network UC instances covering a 50x range in system size. Three primary cases — the 6-bus system [5], IEEE 14-bus [6], and IEEE 39-bus New England system [7,8] — are used for detailed analysis. Four additional cases — IEEE 30-bus, 57-bus, 118-bus, and 300-bus [6] — form a scaling study set. Network topology, line susceptances, thermal limits, and generator capacity limits are taken verbatim from the respective benchmark sources. UC operational parameters for the primary cases are drawn from the 10-unit benchmark of Kazarlis et al. [1], matched by technology type and scaled proportionally to Pmax; nuclear and hydro parameters follow the RTS-GMLC test system [2]. UC parameters for the scaling cases follow the methodology of UnitCommitment.jl [3], setting ramp rates to 30% of Pmax per hour and assigning minimum up/down times by technology class. Renewable units are added at selected buses with diurnal availability profiles following NREL ATB archetypes [4]. All demand profiles are 24-hour normalized load curves scaled to each system's peak.

# 5.  References

[1]  Kazarlis, S.A., Bakirtzis, A.G., & Petridis, V. (1996). A genetic algorithm solution to the unit commitment problem. IEEE Trans. Power Syst., 11(1), 83–92. DOI: 10.1109/59.485989

[2]  Barrows, C., Bloom, A., Ehlen, A., et al. (2019). The IEEE Reliability Test System: A Proposed 2019 Update. IEEE Trans. Power Syst. DOI: 10.1109/TPWRS.2019.2925557  [RTS-GMLC: github.com/GridMod/RTS-GMLC]

[3]  Xavier, A.S., Kazachkov, A.M., Yurdakul, O., He, J., & Qiu, F. (2024). UnitCommitment.jl (v0.4). Zenodo. DOI: 10.5281/zenodo.4269874  [github.com/ANL-CEEESA/UnitCommitment.jl]

[4]  NREL (2023). Annual Technology Baseline. https://atb.nrel.gov/

[5]  Wood, A.J., Wollenberg, B.F., & Sheblé, G.B. (1996). Power Generation, Operation, and Control (2nd ed.). John Wiley & Sons.

[6]  Christie, R.D. (1993). Power Systems Test Case Archive. University of Washington. https://labs.ece.uw.edu/pstca/  |  Zimmerman, R.D., et al. (2011). MATPOWER. IEEE Trans. Power Syst., 26(1), 12–19. DOI: 10.1109/TPWRS.2010.2051168

[7]  Athay, T., Podmore, R., & Virmani, S. (1979). A practical method for the direct analysis of transient stability. IEEE Trans. Power App. Syst., PAS-98(2), 573–584.

[8]  Pai, M.A. (1989). Energy Function Analysis for Power System Stability. Kluwer Academic Publishers.

[9]  IEEE Task Force on Test Systems for Security Assessment (1993). IEEE 300-bus test system. IEEE Power Engineering Society.

[10]  Coffrin, C., et al. (2019). PGLib-OPF: Power Grid Library for Benchmarking AC Optimal Power Flow Algorithms. github.com/power-grid-lib/pglib-opf